Cornell University College of Veterinary Medicine

Student Training Simulation Project

XML Scenario Specification

2026-October-05

Version 1.12

I-Town Design

David Weiner

V: 607 227 2958

E: david@itowndesign.com

\

Terry Kelleher

V: 607-351-6456

E: terry@itowndesign.com

Sales

E: sales@itowndesign.com

Tech Support

E: tech@itowndesign.com

1.	Introduction and Overview	5

2.	Synchronization and Scenario Management	6

2.1	Scenario Upload	6

2.2	Scenario Format	7

2.3	Scenario Images Directory	7

2.4	Scenario Vocals Directory	8

2.5	Scenario Media Directory	9

3.	Schema	10

4.	Scenario Header	12

5.	Scenario Profile	13

6.	Scenario Vocals	16

7.	Scenario Media Files	17

8.	Scenario Init	18

9.	Scenario Events	19

10.	Scenario Scenes	21

10.1	Scenario Schema	23

10.2	Scenario Init	25

10.3	Scenario Timeout	25

10.4	Scenario Triggers	25

10.5	Group Triggers	27

10.6	Trigger Examples	28

11.	API Values	30

11.1	Scenario	30

11.2	Logfile	31

11.3	Cardiac	31

11.4	Respiration	32

11.5	General	32

11.6	Vocals	34

11.7	Media	34

11.8	CPR	34

11.9	Pulse	34

History

2016-June-03 – V 1.0 -- Original Release.

2016-July-14 – V 1.1 – Update descriptions to latest status requests.

2016-July-14 – V 1.2 – Update descriptions for Scenario Init, CPR and Event Triggers

2016-Aug-18 – V 1.3

Updated section 6 to include a new avatar element, color specifier for non vital controls, species and patient image.

Changed specifier for initial scene.

2018-Feb-13 – V 1.4

Updated Sec 2 to describe scenario directories and zip archive format.

2018-July-18 – V 1.5

Various

2019-May-10 – V 1.6

Added description of event hotkeys

2023 – V 1.7

Updated contact information, removed watermark

2023-July-07 – V 1.8

Updated trend-able parameters

2023-July-28 – V 1.9

Updated scenario management description

Updated Sec 10.4 – Scenario Triggers

Updated JSON Schema

Added Sec 10.5 – Trigger groupings

Added Sec 11.9 – Example Trigger Groupings

2024-March-11 – V 1.10

Updated Section 11.3 for cardiac:rhythm API.

2025-April-25 – V 1.11

- This version is supported in SimMgr version 2.2 and above

- Add Improved Group Triggers support.

- Remove the earlier Group Trigger support, implemented in SimMgr version 2.1.

- Add new Trigger Targets

  - cardiac:avg_rate

  - respiration:awRR

2026-October-05 – V 1.12

- Reconciled the entire specification against the parser source (sim-parse.cpp, scenario_xml.cpp, VetSim.cpp). Every element the parser accepts is now documented, and every element documented here was confirmed present in the parser.

- Added the direct arterial pressure parameters: cardiac:abp_line and cardiac:abp_waveform.

- Added the pulmonary artery catheter parameters: cardiac:pac_placed, cardiac:pac_position, cardiac:pac_ra_mean, cardiac:pac_rv_sys, cardiac:pac_rv_dia, cardiac:pac_pa_sys, cardiac:pac_pa_dia and cardiac:pac_wedge_mean.

- Added the waveform morphology parameters: respiration:co2_waveform and respiration:spo2_waveform.

- Added parameters that were implemented but never documented: cardiac:nibp_read, cardiac:nibp_linked_hr, cardiac:nibp_freq, cardiac:pr_interval, cardiac:qrs_interval, cardiac:pwave, cardiac:heart_sound_mute, cardiac:transfer_time, respiration:left_lung_sound_mute, respiration:right_lung_sound_mute, respiration:manual_breath, respiration:transfer_time, general:temperature_units, general:clock_start, general:transfer_time, cpr:duration and the entire telesim class.

- Added Section 11.11, a complete list of the parameters that may be used as trigger targets. This is a strict subset of the API values, which was not previously stated.

- Added Section 12, Known Engine Defects, recording behaviour that differs from this specification and should be avoided when authoring.

- Corrected cardiac:vfib_amplitude – the value is "med", not "medium".

- Corrected cardiac:vpc – the 3-1, 3-2 and 3-3 values are not implemented and are ignored by the engine.

- Corrected respiration:inhalation_duration and respiration:exhalation_duration – these are reported by the engine and cannot be set. The parser entries are commented out.

- Corrected general:temperature – the range is expressed in tenths of a degree, so 0 – 1100, not 0 – 110 as previously stated.

- For some settings an additional tag called “transfer_time” can be added to trend to the specified value(s) over a period of seconds rather than stepping to them immediately.  See Section 11 for the list of parameters that are trendable.

- Corrected the profile button list – the NIBP cuff button id is button-bpcuff, not button-cuff. Added button-pac and button-abp, renamed the Palpate and CPR buttons Pulse and Comps, and documented the sensor-button row: <top> and <left> no longer apply to the seven sensor buttons.

- Removed <triggers_needed> from the scene schema. The element is rejected by the parser and aborts the scenario; use <trigger_group>.

- Noted that <init><record> is not read by the engine.

Cornell University College of Veterinary Medicine

Student Training Simulation Project

XML Scenario Specification

Introduction and Overview

This document describes the XML Scenario Specification for the Cornell University College of Veterinary Medicine Student Training Simulation Project.

A scenario is used to manage mannequin actions and vital signs based on student responses to presented conditions.  A scenario is composed of the following major components.  Each component is described in detail further in this document:

- Scenario XML Schema – The schema for the XML document will be proprietary and described in this document.  The syntax will follow established rules for well-formed XML structures.

- Scenario Header – The header is used to provide descriptive information about the scenario such as time of creation, scenario name, author, etc.

- Scenario Profile – The profile is used to specify which animal image should be used on the instructor interface, patient description, placement of auxiliary controls, and other profile specific information.

- Scenario Vocal Files – The vocal files are used by the instructor interface to play animal sounds that a patient may produce during an examination such as a dog growl or cat hiss.

- Scenario Media Files – The media files are displayed on the student interface to provide additional diagnostic information about a patient such as MRI’s, X-Rays, or other graphic information.

- Scenario Init – The init section is used to preset specific scenario conditions to a value other than the default value.  The values in the init section are set before the scenario is started.

- Scenario Events – The events are scenario specific and may be selected when an action is performed by the students that represent the event (ie injecting a drug).  Specific events may trigger an exit condition from a scenario scene.  All events are logged in the event monitor.  Events are grouped together with defined categories.

- Scenario Scenes – Each scenario will be made up of scenes which will present a set of conditions to the students.  Each scene will contain entry conditions and exit conditions.  The exit conditions will include actions such as an injection, CPR, administrating an AED shock, timeout form inaction or other event.  Any parameter can be set within a scene including trends.

Synchronization and Scenario Management

There are three main components to the Simulator:

- Instructor Interface (II)

- Simulation Manager (SimMgr)

- Simulation Controller / Mannequin (SimCtrl)

The Simulation Manager is the main controller of the system.  When all components are booting up, the SimMgr will attempt communication to the other two components.  Upon a successful synchronization, a default scenario will be loaded and communicated to the II and SimCtrl.  The II will then be able to tell the SimMgr that a new scenario has been selected, thus initiating a sequence of loading the new XML, setting up all vital signs and auxiliary controls, then beginning the scenario with the first scene.

The II will present a list of available scenarios to the instructor and provide feedback on the current scenario such as run time and current scene.

All scenario XML, profile images, and media files will be saved in predetermined directories.

Scenario Upload

Scenarios can be uploaded using the “Scenarios” menu item in the Instructor Interface.

When clicked, the interface will provide a list of available scenarios with an option to delete.

The scenario summary shows the scenario directory the scenario can be found in, the scenario name as read from the xml file <header><title><name> section, the description found in the <header><description> section, and the date created found in the <header><date_of_creation> field.

The “Delete Scenario” link will delete that scenario

New scenarios can be added, or existing scenarios can overwritten by navigating to the scenario directory.  A desktop icon is available to go to this directory:

Scenario Format

A scenario must have a specific format:

- There can only be one xml file.  The file is expected to be called ‘main.xml’.  If it is not called ‘main.xml’, it will be renamed.

- There must be three directories:  images, vocals, and media.

Scenario Images Directory

The scenario “images” directory will contain up to two files:  The avatar file as specified in <profile><avatar><filename>.  The second file is the image used for the summary presented at the beginning of a scenario:  <profile><summary><image>

Scenario Vocals Directory

The scenario “vocals” directory will contain the vocalization files as specified in the <vocals><file><filename> section of the scenarios.  There is no minimum or maximum number of files for this section, but the directory must exist.  The actual files must be in a .wav format and will be made available under the “Vocalizations” function found in the avatar section.

Scenario Media Directory

The scenario “media” directory will contain the media files as specified in the <media><file><filename> section of the scenarios.  There is no minimum or maximum number of files for this section, but the directory must exist.  The actual files can be in any recognized portable format such as image, pdf, or other document format recognized by the OS browser.

Schema

The schema will be proprietary and follow the rules for well-formed XML structures.  The basic schema will be as follows:

<?xml version=”1.0” encoding=”UTF-8”?>

<scenario>

<header>

.

.

.

</header>

<profile>

.

.

.

</profile>

<vocals>

.

.

.

</vocals>

<media>

.

.

.

</media>

<init>

.

.

.

</init>

The initial scene may be written as either <initial_scene>N</initial_scene> or <scene>N</scene>; the parser accepts both and treats them identically.

NOTE:  <record> is accepted in the init section by convention and appears in the supplied scenarios, but the engine does not read it.  Recording is controlled from the instructor interface.  See Section 12.

<events>

.

.

.

</events>

<scene>

.

.

.

</scene>

<scene>

.

.

.

</scene>

</scenario>

There will be only one header, profile, vocals, media, init, and events elements.  There will be one or more scene elements.  The details for each element will be presented below.

Scenario Header

The header is used to provide metadata or descriptive information about the scenario.  The header will have the following elements:

There is one header section in the XML.

<header>

<author>Terry Kelleher</author>

<title>

<name>Healthy 40kg Dog </name>

<top>5</top> // Position from top of page in pixels

<left>10</left> // Position from left of page in pixels

</title>

<date_of_creation>2016-06-03</date_of_creation >

<description>

This is a description of the scenario.  It can be a very long description or a short description.  Final format will depend on how information is displayed, if at all.

</description>

</header>

Scenario Profile

The scenario profile is used to specify the background image name, summary information about the patient, placement of the non-vital controls (i.e. vocals, lung sounds, etc.), and available vocalizations.  The image filename can specify any portable media format (i.e. jpg, gif, etc.).  Vocalizations are assumed to be .wav format.

The avatar section specifies the background image used for the non-vital controls.  The height_pct and width_pct elements specify the rendered size of the image for the background in a percentage from 0 – 100.

The color element in the control section specifies the rendered color for the non-vital control labels.  Any HTML5 compatible specifier can be used such as ‘#000000’ for black, ‘#FFFFFF’ for white, or any web safe color name.

There are currently 9 controls available which when clicked will open a control modal.  Each control can be specified with any title, but the id must be exactly specified as listed below:

- Vocalizations (id: vocals-dog-control) – clicking on this control will open the vocalizations modal.

- Right Lung Sound (id: right-lung-dog-control) – clicking on this control will open the right lung sound modal.

- Left Lung Sound (id: left-lung-dog-control) – clicking on this control will open the right lung sound modal.

- Left Femoral Pulse (id: left-femoral-pulse-dog-control) – clicking on this control will open the left femoral pulse control modal.

- Right Femoral Pulse (id: right-femoral-pulse-dog-control) – clicking on this control will open the right femoral pulse control modal.

- Left Dorsal Pulse (id: left-dorsal-pulse-dog-control) – clicking on this control will open the left dorsal pulse control modal.

- Right Dorsal Pulse (id: right-dorsal-pulse-dog-control) – clicking on this control will open the right dorsal pulse control modal.

- Heart Sounds (id: heart-sound-dog-control) – clicking on this control will open the heart sound control modal.

- Chest Movement (id: chest -dog-control) – clicking on this control will open the chest movement control modal.

There are 9 buttons available which will either activate a function or indicate a specific response.  These are listed below:

- Comps (id: button-cpr) – this button will indicate when chest compressions (CPR) are detected.

- ECG (id: button-ekg) – when clicked this button will activate the ECG reading on the student monitor.

- SpO2 (id: button-SpO2) – when clicked this button will activate the SpO2 reading on the student monitor.

- ETCO2 (id: button-CO2) – when clicked this button will activate the ETCO2 reading on the student monitor.

- Cuff (id: button-bpcuff) – when clicked this button will activate the NIBP reading on the student monitor.  NOTE: Once activated the “Read NIBP” button still needs to be clicked to take a reading.

- Pulse (id: button-palpate) – this button indicates when the students palpate a pulse point.  The button will glow either yellow (light pressure), green (normal pressure), or red (hard pressure).  The associated pulse control will also be highlighted when palpated.

- Temp (id: button-Tperi) – when clicked this button will activate the Tperi reading on the student monitor.

- PA Catheter (id: button-pac) – when clicked this button places or removes a pulmonary artery (Swan-Ganz) catheter.  Placing one adds the PA catheter waveform to the patient monitor; removing it takes the waveform away and the remaining waveforms expand to fill the space.

- ABP (id: button-abp) – when clicked this button places or removes a direct (invasive) arterial line.  Placing one adds the arterial pressure waveform, with its systolic, diastolic and mean pressures, to the patient monitor; removing it takes the waveform away and the remaining waveforms expand to fill the space.  It sets the same value as cardiac:abp_line.

NOTE:  Any of the nine controls listed first (vocalizations, lung sounds, pulses, heart sounds and chest movement) can be hidden by removing its definition from the scenario file.  Buttons cannot be hidden this way.

NOTE:    The left and right dorsal pulse points are currently under development.

NOTE:    Pulse and Comps are placed on the avatar by the <top> and <left> of their <control> entries, as the controls are.  The seven sensor buttons – ECG, SpO2, Temp, PA Catheter, ABP, ETCO2 and Cuff – are laid out by the instructor interface in a fixed row beneath the avatar, in that order.  Their <control> entries still set each button's label through <title>, but <top> and <left> are ignored.  A sensor button with no <control> entry still appears, with its default label.

There is one profile section in the XML file.

<profile>

<avatar>

<filename>stock-dog.jpg</filename>

<height_pct>100</height_pct>

<width_pct>50</width_pct>

</avatar>

<summary>

<description>

Lorem ipsum dolor sit amet, consectetur adipiscing elit. Vestibulum aliquet lectus id ante aliquet pharetra. Proin quis neque nisi. Integer sed odio congue, bibendum purus id, imperdiet leo. Morbi tempus nibh ac malesuada egestas. Quisque facilisis, purus suscipit tincidunt bibendum, purus leo auctor mauris, ac ullamcorper diam odio eget nulla. Mauris dignissim, lacus dapibus lobortis dignissim, odio velit volutpat nulla, ut convallis nisi urna eu nulla. Sed interdum nisi in orci ullamcorper tincidunt. Aliquam erat volutpat. Sed id efficitur tortor, tincidunt condimentum urna.

</description>

<breed>German Shepard</breed>

<gender>Male</gender>

<weight>35 kg</weight>

<species>Canine</species>

<symptoms>

Not eating, no energy, excessive panting.

</symptoms>

<image>case_12345.jpg</image>

</summary>

<controls>

<color>#000000</color>

<control>

<title>Vocals</title>

<id>vocals-dog-control</id>

<top>45</top>

<left>405</left>

</control>

<control>

<title>Right Lung Sounds</title>

<id>right-lung-dog-control</id>

<top>160</top>

<left>135</left>

</control>

*

*

*

</controls>

</profile>

Scenario Vocals

The scenario vocals section provides the ability to download different animal sounds which might be used during a scenario (ie cat hiss, dog growl).  There can be none or unlimited number of vocal files.  The vocal files should be in a recognized audio format such as wav or mp3.

There is one vocal section in the scenario XML.

<vocals>

<file>

<filename>bark.wav</filename >

<title>Barking</title>

</file>

<file>

<filename >growl.wav</filename >

<title>Growling</title>

</file>

*

*

*

</vocals>

Scenario Media Files

The scenario media files are used to visualize different images on the student monitor under instructor control.  Files can be any displayable format recognized by browsers including non-image files such as a pdf or a Word document.

There is one media section in the scenario XML.

<media>

<file>

<filename >mri.jpg</filename >

<title>MRI of healthy dog</title>

</file>

<file>

<filename >x-ray.pdf</filename >

<title>X-Ray of healthy dog</title>

</file>

<file>

<filename >blood_work.jpg</filename >

<title>Blood work of healthy dog</title>

</file>

*

*

*

</media>

Scenario Init

The init section is used to initialize parameters when the scenario is loaded.  The init will have initial values for any selected signals, the initial scene to be executed once the scenario is started, and a preset for the video record setting.

There is one init section in the scenario XML.  Any parameter listed in Section 11 may be set here, grouped under its signal group.  In addition, <initial_scene> selects the scene the scenario starts in.

<init>

<cardiac>

*

*

*

</cardiac>

<respiration>

*

*

*

</respiration>

*

*

*

<initial_scene>1</initial_scene>

<record>1</record>

</init>

Scenario Events

Scenario events are the list of events that can be manually selected by the instructor during the training session.  When an event is selected it is entered into the log and used as a potential trigger for the current scene.  The event id is used to identify the selected event.  All event ID’s must be unique.  The events are listed under the specified category when the instructor selects ‘Events’ from the top menu.

Specific events can be selected as ‘Priority’.  If the priority element is added with a value of one (‘1’), the event will be listed in the top navigation area as a convenience link for the instructor.

An optional hotkey can be added for an event.  The hotkey can only be a single character.

There is one events section in the XML file.  The number of events is not limited.

<events>

<category>

<name>drugs</name>

<title>Injected Drugs</title>

<event>

<title>Morphine</title>

<id>opiate_morphine</id>

<priority>1</priority>

<hotkey>a</hotkey>

</event>

<event>

<title>Vicodin</title>

<id>hydrocodone</id>

</event>

*

*

*

</category>

<category>

<name>abc</name>

<title>ABC</title>

<event>

<title>Intubate</title>

<id>intubate</id>

</event>

<event>

<title>CPR</title>

<id>cpr</id>

</event>

<event>

<title>Shock</title>

<id>aed</id>

</event>

*

*

*

</category>

*

*

*

</events>

Scenario Scenes

Scenario scenes are used to control responses of the mannequin to student actions.  There can be multiple scenes associated with a scenario.  Each scene is divided into a title, id, a set of initialization conditions, and a set of optional triggers, events and timeouts used to branch to other scenes.

The title and id are used to uniquely identify the scene on the instructor interface.  All scene ID’s are integer values and must be unique. The scene ID’s are also used to identify which scene to jump to for a timeout or trigger.

All initialization parameters match the syntax used for the API.  To see an example of the API go to the following URL:

All initialization parameters match the syntax used for the API.  The full status of a running simulator can be read from the SimMgr status endpoint:

http://localhost:50002/cgi-bin/simstatus.cgi?status=1http://vet.newforce.us/cgi-bin/simstatus.cgi?status=1

An example status return in JSON format is shown below:

"cardiac" : {

"rhythm":"sinus",

"vpc":"none",

"pea":"0",

"vpc_freq":"0",

"vpc_delay":"0",

"vfib_amplitude":"high",

"rate":"80",

"avg_rate":"80",

"nibp_rate":"80",

"nibp_read":"-1",

"nibp_linked_hr":"1",

"nibp_freq":"0",

"pulseCount":"36",

"pulseCountVpc":"0",

"pwave":"none",

"pr_interval":"140",

"qrs_interval":"85",

"bps_sys":"105",

"bps_dia":"70",

"right_dorsal_pulse_strength":"medium",

"left_dorsal_pulse_strength":"medium",

"right_femoral_pulse_strength":"medium",

"left_femoral_pulse_strength":"medium",

"heart_sound_volume":"10",

"heart_sound_mute":"0",

"heart_sound":"none",

"ecg_indicator":"0",

"bp_cuff":"0",

"arrest":"0"

}

"respiration" : {

"left_lung_sound":"normal",

"left_lung_sound_volume":"10",

"left_lung_sound_mute":"0",

"right_lung_sound":"normal",

"right_lung_sound_volume":"10",

"right_lung_sound_mute":"0",

"inhalation_duration":"1050",

"exhalation_duration":"1050",

"breathCount":"9",

"spo2":"95",

"etco2":"34",

"rate":"20",

"awRR":"20",

"etco2_indicator":"0",

"spo2_indicator":"0",

"chest_movement":"0",

"manual_count":"0"

},

"general" : {

"temperature":"1017",

"temperature_units":"F",

"temperature_enable":"0"

},

"pulse" : {

"right_dorsal":"0",

"left_dorsal":"0",

"right_femoral":"0",

"left_femoral":"0",

"duration":"0",

"active":"0"

},

NOTE:  Not all parameters are writeable.  Please see section 11 for full details.

To specify a new heart rhythm, the following XML elements would be used:

<cardiac>

<rhythm>afib</rhythm>

</cardiac>

For some settings, an additional tag called “rate_trend” can be added in to specify a trend over a period of seconds to the specified value(s).  See section 11 for a list of parameters that are trendable.

See section 11 for a complete description of the values that can be used for scenarios.

Scenario Schema

A scenario scene will follow the specified format shown below:

<scene>

<title>Displayed title of scene</title>

<id>ID of scene (unique integer)</id>

<init>

Parameters to be initialized per API specification

</init>

<timeout>

<timeout_value>Timeout value in seconds</timeout_value>

<scene_id>

ID of scene to jump to if scene timeouts (ie no trigger)

</scene_id>

</timeout>

<triggers>

<trigger>

*

*

*

</trigger>

<trigger>

*

*

*

</trigger>

<trigger>

*

*

*

</trigger>

</triggers>

</scene>

The title and ID are used to identify the scene.

### Scene ID:

- The scene ID must be a unique Integer Value

- A value of 0 (zero) or a Negative value will indicate that the scene is an ending scene. The simulation scenario will automatically terminate

Scene Title:

- The scene Title is a label that will be shown on the II.

- An overly long title might cause overflows of text within the II header.  The title is truncated to 32 characters by the parser.

- Removed: triggers_needed

- Earlier versions of this specification described a <triggers_needed> element at the scene level.  It has been removed.  The parser now rejects it, reports an error and refuses to load the scenario.  Use <trigger_group> with <triggers_required> instead – see Section 10.5.

Scenario Init

The scenario init is used to set parameters before the scenario is executed.  The parameters are specified using the API as specified in Section 11

Scenario Timeout

A scenario timeout is used to force a jump to another scene if none of the triggers are detected as follows:

<timeout>

<timeout_value>timeout in seconds</timeout_value>

<scene_id>id of scene to jump to</scene_id>

</timeout>

Scenario Triggers

Each scene within a scenario can specify any number of triggers.  Triggers are used to specify a condition to force a jump to a new scene.  There are three types of triggers:

CPR trigger, event trigger, and a parameter meeting a test criteria.  Triggers may also be grouped to force multiple triggers to occur before a branch is taken.

A scene can also trigger on palpation of the femoral pulse, palpation duration, connecting a probe, and manual breaths.

Trigger Test Conditions

CPR and Parameter Triggers can use an optional test condition.  This test condition allows a comparison of a dynamic comparison of a value to a test condition to determine if a trigger has occurred.  The conditions are as follows:

EQ = specified parameter

GT > specified parameter

LT < specified parameter

LTE <= specified parameter

GTE >= specified parameter

INSIDE >= specified parameter && <= specified parameter

OUTSIDE < specified parameter || > specified parameter

The test condition is compared without case folding and must be written in upper case.  An unrecognised condition silently falls back to EQ.

INSIDE and OUTSIDE take two bounds written as low-high with no spaces, for example <rate>60-120</rate>.  Because the bounds are split on the first hyphen, negative bounds cannot be expressed.

There is no EVENT test condition.  An event trigger is written with <event_id> and no <test> element; the parser sets the condition itself.

Examples are provided below showing usage.

CPR Trigger

The CPR trigger has the following format:

<trigger>

<test>Test Condition</test>

<scene_id>Scene ID to jump to</scene_id>

<cpr>

<duration>Duration in seconds</duration>

</cpr>

</trigger>

Where Test Condition is described in section 10.4.1, the scene ID is the unique identifier of the scene to jump to and the duration is the amount of time in seconds that there is detected CPR activity.

Event Trigger

An event trigger will respond to an event being logged by the instructor.  When the event is logged, the scenario will branch as specified in the trigger.  The event trigger has the following format:

<trigger>

<event_id>id of event as specified in scenario</event_id>

<scene_id>new scene id</scene_id>

</trigger>

Parameter Trigger

The parameter trigger is used to branch when a monitored parameter reaches a threshold value.  The format of the parameter trigger is as follows:

<trigger>

<test>Test Condition</test>

<scene_id>Scene ID to jump to</scene_id>

<Signal Group>

<Signal Name>Value </ Signal Name>

</ Signal Group >

</trigger>

Where test condition is as described in section 10.4.1 scene ID is the scene to jump to if the trigger conditions are met, and the signal group and signal name are as described in section 11.

Group Triggers

Trigger Groups may be specified to allow triggering on a combination of events and parameter conditions. A scene may have multiple Trigger Groups defined.

The Trigger Groups are defined within the <triggers> section of the scene.

The following format is used to implement a group trigger:

<trigger_group>

<group_id>1</group_id >

<scene_id>3</scene_id >

<triggers_required>2</triggers_required>

<trigger>

</trigger>

<trigger>

</trigger>

<trigger>

</trigger>

</trigger_group>

<trigger_group>

<group_id>2</group_id >

<scene_id>3</scene_id >

<triggers_required>1</triggers_required>

<trigger>

</trigger>

<trigger>

</trigger>

<trigger>

</trigger>

</trigger_group>

Trigger Examples

Create a trigger that will branch to scene ID 12345 if CPR is detected for at least 30 seconds.

<trigger>

<test>GTE</test>

<scene_id>12345 </scene_id>

<cpr>

<duration>30</duration>

</cpr>

</trigger>

Create a trigger that will branch to scene ID 4 if the event “Defib” is logged by the instructor.

<trigger>

<event_id>Defib</event_id>

<scene_id>4</scene_id>

</trigger>

Create a trigger that will branch to scene id 5 if the heart rate goes under 10 bpm.

<trigger>

<test>LT</test>

<scene_id>5 </scene_id>

<cardiac>

<rate>10</rate>

</cardiac>

</trigger>

Create a trigger using INSIDE range for awrr between 10 and 20 bpm.

<trigger>

<test>INSIDE</test>

<scene_id>6 </scene_id>

<respiration>

<awrr>10-20</awrr>

</respiration>

</trigger>

Create a trigger where CPR needs to be performed for at least 60 seconds with 4 manual breaths:

<trigger_group>

<scene_id>12345 </scene_id>

<triggers_required>2</triggers_required>

<trigger>

<test>GTE</test>

<cpr>

<duration>60</duration>

</cpr>

</trigger>

<trigger>

<test>GTE</test>

<respiration>

<manual_count>4< /manual_count >

</respiration>

</trigger>

</trigger_group>

Create a trigger where CPR needs to be performed for at least 2 minutes seconds and an injection of either Epinephrine or Vasopressin is required. In this example, two trigger groups are defined.

<triggers>

<trigger_group>

<scene_id>2 </scene_id>

<triggers_required>2</triggers_required>

<trigger>

<test>GTE</test>

<cpr>

<duration>120</duration>

</cpr>

</trigger>

<trigger>

<event_id>epinephrine</event_id>

</trigger>

</trigger_group>

<trigger_group>

<scene_id>2 </scene_id>

<triggers_required>2</triggers_required>

<trigger>

<test>GTE</test>

<cpr>

<duration>120</duration>

</cpr>

</trigger>

<trigger>

<event_id>vasopressin</event_id>

</trigger>

</trigger_group>

</triggers>

API Values

The following main categories are used for the API signal groups:

- Scenario – used to specify what scenario is being run, the state of the scenario, runtime, sceneID, and if the scenario is being recorded.

- Logfile – specifies the current logfile and record pointer.

- Cardiac – specifies all parameters associated with cardiac simulation.

- Respiration – specifies all parameters associated with respiration.

- General – specifies temperature.

- Vocals – specifies if a vocalization file is being played.

- Media – specifies if a media file is being displayed.

- CPR – specifies whether CPR has been detected, with measured depth of compression and of release, and the accumulated compression time in the current scene.

- Pulse – reports which pulse points are being palpated, how hard, and for how long.

- Telesim – controls the remote video windows.

To specify a value in the xml, use the signal group, then the signal name.  For example to specify etco2 create the following tags:

<respiration>

<etco2>50<etco2>

</respiration>

Some parameters can have a trend by adding in “transfer_time” as follows:

<respiration>

<etco2>50<etco2>

<transfer_time>100</transfer_time>

</respiration>

Parameters that are trendable are marked with a (T).  A trend is armed by the transfer_time in the same signal group, so one transfer_time governs every trended parameter set in that group.  There are exactly three transfer_time values: cardiac, respiration and general.

Parameters marked (S) are reported by the engine and cannot be set.  A value written to one of these in a scenario is silently discarded.

Parameters marked (X) are accepted by the parser but are not applied by the engine.  They are listed for completeness and should not be used.  See Section 12.

Not every parameter listed here can be used as a trigger target.  Section 11.11 gives the definitive list of trigger targets.

Details of parameters are specified in the following sections:

Scenario

| scenario:active | Name of scenario selected. It is assumed that name is the base name of the scenario file. For example ‘test’ would mean that file test.xml is being selected. |
| --- | --- |
| scenario:start | Timestamp of when scenario began. |
| scenario:runtime | Runtime in format hh:mm:ss |
| scenario:scene_name | Name of current scene |
| scenario:scene_id | ID of current scene |
| scenario:state | State of current scenario: {Stopped ¦ Paused ¦ Terminate ¦ Running } |

Logfile

| logfile:active | Specify if current logfile is active. {0 = inactive ¦ 1 = active} |
| --- | --- |
| logfile:filename | Name of current logfile |
| logfile:lines_written | Pointer to current record |

Cardiac

| cardiac:rhythm | Specify selected rhythm of heart ECG. {sinus ¦ afib ¦ vfib ¦ vtach1 ¦ vtach2 ¦ vtach3 ¦ asystole} NOTE: vtach3 will initiate an R on T ECG waveform. |
| --- | --- |
| cardiac:vpc | Specify if a VPC is selected. {none ¦ 1-1 ¦ 1-2 ¦ 1-3 ¦ 2-1 ¦ 2-2 ¦ 2-3}. The first digit is the VPC type: 1 = vtach1, 2 = vtach2. The second digit is the number of VPC pulses: 1 = singlet, 2 = couplet, 3 = triplet. NOTE: type 3 (R on T) is not implemented. The values 3-1, 3-2 and 3-3 are accepted by the parser and then ignored. |
| cardiac:vpc_freq | Specify how often a VPC will be inserted after a sinus complex. 0 – 100, in increments of 10, as a percentage of normal sinus pulses. |
| cardiac:vpc_delay (X) | Accepted by the parser but never applied – the code that would transfer it is disabled. Do not use. |
| cardiac:pea | Specify pulseless electrical activity. {1 = on ¦ 0 = off} |
| cardiac:arrest | Specify if the arrest indicator is illuminated on the instructor interface. {0 = not illuminated ¦ 1 = illuminated} |
| cardiac:vfib_amplitude | Specify the amplitude of the ventricular fibrillation signal. {low ¦ med ¦ high} NOTE: the value is "med". "medium" is not recognised. |
| cardiac:pwave | Stored by the engine and reported in the status. Reserved; it has no effect on the displayed waveform in the current release. |
| cardiac:rate (T) | Specify heart rate. Range 0 – 300 bpm. |
| cardiac:avg_rate (S) | Calculated heart rate, averaged by the engine. Read only; usable as a trigger target. |
| cardiac:transfer_time | Trend duration in seconds for cardiac:rate, cardiac:bps_sys and cardiac:bps_dia. A value of 0, or no transfer_time at all, applies the new value immediately. The instructor interface offers 0, 20, 40, 60, 120, 180, 240, 300, 360, 480 and 600 seconds. |
| cardiac:pr_interval | PR interval in milliseconds. Default 140. |
| cardiac:qrs_interval | QRS interval in milliseconds. Default 85. |
| cardiac:bps_sys (T) | Specify systolic blood pressure. 0 – 300 mmHg. Also drives the systolic value on the direct arterial pressure waveform. |
| cardiac:bps_dia (T) | Specify diastolic blood pressure. 0 – 290 mmHg. Also drives the diastolic value on the direct arterial pressure waveform. |
| cardiac:nibp_rate | Specify the pulse rate reported alongside a non-invasive blood pressure reading. |
| cardiac:nibp_read | Set to 1 to start a non-invasive blood pressure read. The engine sets it back to 0 when the read completes. |
| cardiac:nibp_linked_hr | If 1, the reported NIBP pulse rate follows the heart rate. If 0, cardiac:nibp_rate is reported instead. Default 1. |
| cardiac:nibp_freq | Interval in minutes between automatic NIBP reads. 0 = manual reads only. Default 0. |
| cardiac:ecg_indicator | Specify if the ECG indicator is illuminated, indicating that the ECG leads have been attached. {0 = not illuminated ¦ 1 = illuminated} On the student monitor this also gates whether the ECG waveform is drawn. |
| cardiac:bp_cuff | Specify if the blood pressure cuff is attached. {0 = not attached ¦ 1 = attached} |
| cardiac:abp_line | Specify if a direct (invasive) arterial line has been placed and the transducer zeroed. {0 = not placed ¦ 1 = placed} With the line placed, the patient monitor shows a continuous arterial pressure waveform with systolic, diastolic and calculated mean pressures. Without it, the arterial waveform and its numbers are removed from both the student monitor and the instructor interface, and the remaining waveforms expand to fill the space. The instructor can also place and remove the line with the ABP button (button-abp) in the sensor row; both set this value. |
| cardiac:abp_waveform | Morphology of the arterial pressure waveform. {normal ¦ overdamped ¦ underdamped ¦ poor ¦ cpr} normal, poor and cpr are physiological states: the monitor reports the systolic and diastolic you set in cardiac:bps_sys and cardiac:bps_dia, and only the shape of the pulse changes. overdamped and underdamped are measurement artifacts: the displayed pressures deviate from the values you set, which is the teaching point. For a set pressure S/D, overdamped displays approximately D+0.76(S−D) over D+0.14(S−D), and underdamped approximately D+1.41(S−D) over D−0.06(S−D). A set 120/80 therefore reads 110/86 overdamped and 136/78 underdamped. cpr free-runs at 110 compressions per minute rather than following the ECG. It is instructor-selected and does not engage automatically. |
| cardiac:pac_placed | Specify if a pulmonary artery (Swan-Ganz) catheter is in the patient. {0 = not placed ¦ 1 = placed} With a catheter placed, the patient monitor gains a fifth waveform channel and the other channels are re-divided to make room. Removing it returns the space. |
| cardiac:pac_position | Where the catheter tip is sitting. {cvp ¦ ra ¦ rv ¦ pa ¦ wedge} These are the positions a catheter is floated through, in order, and each produces a distinct pressure waveform: a low-amplitude venous trace in the vena cava (cvp) and right atrium (ra); a trace whose diastole falls to baseline in the right ventricle (rv); a trace with the same systolic peak but a diastolic step-up and a dicrotic notch in the pulmonary artery (pa); and a damped, low-amplitude trace when the balloon is wedged (wedge). The monitor label changes with the position: CVP, RA, RV, PA or PAWP. |
| cardiac:pac_ra_mean | Right atrial / central venous mean pressure in mmHg. Reported when the tip is at cvp or ra. Default 5. |
| cardiac:pac_rv_sys | Right ventricular systolic pressure in mmHg. Default 25. |
| cardiac:pac_rv_dia | Right ventricular end-diastolic pressure in mmHg. Default 5. The trace dips below this in early diastole, which is the feature that identifies the right ventricle. |
| cardiac:pac_pa_sys | Pulmonary artery systolic pressure in mmHg. Default 25. Normally equal to cardiac:pac_rv_sys, because the pulmonic valve is open at peak ejection. |
| cardiac:pac_pa_dia | Pulmonary artery diastolic pressure in mmHg. Default 12. The step up from right ventricular diastole is how the operator knows the pulmonic valve has been crossed. |
| cardiac:pac_wedge_mean | Pulmonary artery wedge (occlusion) mean pressure in mmHg. Default 9. Normally below cardiac:pac_pa_dia. |
| cardiac:heart_sound | Specify the heart sound to be played. {normal ¦ systolic_murmur ¦ pansystolic_murmur ¦ holosystolic_murmur ¦ continuous_murmur ¦ diastolic_murmur ¦ gallop} NOTE: only normal and systolic_murmur are enabled in the instructor interface in the current release. The engine default is "none", which is not one of the selectable values. |
| cardiac:heart_sound_volume | Specify the volume of the heart sound generated in the mannequin. 0 – 10. |
| cardiac:heart_sound_mute | Mute the heart sound at the instructor interface. {0 = not muted ¦ 1 = muted} |
| cardiac:left_dorsal_pulse_strength | Specify left dorsal pulse strength simulated in the mannequin. {none ¦ weak ¦ medium ¦ strong} NOTE: any other value is rejected and the current strength is left unchanged. |
| cardiac:right_dorsal_pulse_strength | Specify right dorsal pulse strength simulated in the mannequin. {none ¦ weak ¦ medium ¦ strong} |
| cardiac:left_femoral_pulse_strength | Specify left femoral pulse strength simulated in the mannequin. {none ¦ weak ¦ medium ¦ strong} |
| cardiac:right_femoral_pulse_strength | Specify right femoral pulse strength simulated in the mannequin. {none ¦ weak ¦ medium ¦ strong} |
| cardiac:pulseCount (S) | Monotonic count of pulses generated. Read only; used by the interfaces to synchronise the waveforms to the heartbeat. |
| cardiac:pulseCountVpc (S) | Monotonic count of VPC pulses generated. Read only. |

Respiration

| respiration:left_lung_sound | Specify the selected sound for the left lung. {normal ¦ coarse_crackles ¦ fine_crackles ¦ wheezes ¦ stridor ¦ stertor} NOTE: stridor and stertor are present but disabled in the instructor interface in the current release. The instructor interface also offers "same_as_right" for the left lung; it is a convenience for the operator and should not be written into scenario XML. |
| --- | --- |
| respiration:right_lung_sound | Specify the selected sound for the right lung. {normal ¦ coarse_crackles ¦ fine_crackles ¦ wheezes ¦ stridor ¦ stertor} |
| respiration:left_lung_sound_volume | Specify the volume for the left lung sound. 0 – 10. |
| respiration:right_lung_sound_volume | Specify the volume for the right lung sound. 0 – 10. |
| respiration:left_lung_sound_mute | Mute the left lung sound. {0 = not muted ¦ 1 = muted} NOTE: the engine default is 1 (muted) for the left lung and 0 for the right. Set both explicitly if it matters. |
| respiration:right_lung_sound_mute | Mute the right lung sound. {0 = not muted ¦ 1 = muted} Engine default 0. |
| respiration:rate (T) | Specify respiration rate. 0 – 60 breaths per minute. |
| respiration:spo2 (T) | Specify SpO₂ value. 0 – 100 %. |
| respiration:etco2 (T) | Specify etCO₂ value. 0 – 150 mmHg. |
| respiration:transfer_time | Trend duration in seconds for respiration:rate, respiration:spo2 and respiration:etco2. A value of 0, or no transfer_time at all, applies the new value immediately. |
| respiration:co2_waveform | Morphology of the capnograph waveform. {normal ¦ rebreathing ¦ obstructive ¦ curare} rebreathing raises the inspired baseline above zero; obstructive produces the upsloping "shark fin" plateau of airway obstruction; curare produces the mid-plateau cleft of partial neuromuscular blockade. |
| respiration:spo2_waveform | Morphology of the pulse oximeter plethysmograph. {normal ¦ poor ¦ artifact} normal and poor are real pulses drawn at full and reduced amplitude. artifact means no usable signal – no detectable pulse, or a probe placement that cannot read – and is non-pulsatile. |
| respiration:etco2_indicator | Specify if the etCO₂ monitor has been connected. The etCO₂ image will be lit green. {0 = not connected ¦ 1 = connected} On the student monitor this also gates whether the capnograph waveform is drawn. |
| respiration:spo2_indicator | Specify if the SpO₂ monitor has been connected. The SpO₂ image will be lit green. {0 = not connected ¦ 1 = connected} On the student monitor this also gates whether the plethysmograph is drawn. |
| respiration:chest_movement | Specify if chest movements will be shown in the mannequin. {0 = off ¦ 1 = on} |
| respiration:manual_breath | Register a manual (bagged) breath. Each value written increments respiration:manual_count. NOTE: any value of 0 or greater counts as a breath, including 0. There is no way to write this parameter without registering a breath. |
| respiration:manual_count (S) | Count of manual breaths detected within the current scene. Reset at the beginning of each scene. Read only – the parser accepts it but the value is never applied. It is a trigger target. |
| respiration:awRR (S) | Airway respiration rate, calculated by the engine from the capnograph. Read only; usable as a trigger target. |
| respiration:breathCount (S) | Monotonic count of breaths generated. Read only. |
| respiration:inhalation_duration (S) | Inhalation time in milliseconds, calculated by the engine from the respiration rate. Read only. NOTE: earlier versions of this specification listed this as settable. It is not – the parser entry is disabled. |
| respiration:exhalation_duration (S) | Exhalation time in milliseconds, calculated by the engine from the respiration rate. Read only. NOTE: earlier versions of this specification listed this as settable. It is not – the parser entry is disabled. |

General

| general:temperature (T) | Specify current temperature. The value is always expressed in tenths of a degree: 1017 = 101.7 degrees, 975 = 97.5 degrees, 800 = 80.0 degrees. Range 0 – 1100. NOTE: earlier versions of this specification gave the range as 0 – 110, which contradicted the examples in the same table. |
| --- | --- |
| general:temperature_units | Units the temperature is displayed in. {F ¦ C} Only the first character is examined, so "Fahrenheit" and "F" behave identically. Any other value is silently ignored. There is no engine default: until a scenario or the instructor sets it the field is empty and the display falls back to Fahrenheit. |
| general:temperature_enable | Specify if the temperature probe has been connected. The temperature icon will be illuminated. {0 = off ¦ 1 = on} |
| general:transfer_time | Trend duration in seconds for general:temperature. |
| general:clock_start (X) | Accepted by the parser, but the engine overwrites it with the current wall-clock time immediately afterwards. Writing any value therefore resets the displayed clock to now rather than starting it at the time given. See Section 12. |

Vocals

| vocals:filename | Specify the filename of the vocalization to be played. The filename includes base and extension and is assumed to be in the scenario’s vocals directory. NOTE: the filename is truncated to 63 characters on its way to the player. |
| --- | --- |
| vocals:repeat | Specify if the vocalization should be repeated. {0 = no repeat ¦ 1 = repeat} |
| vocals:volume | Specify the volume the vocalization should be played at. 0 – 10. |
| vocals:play | Initiate playing of the vocalization file. {0 = do not play ¦ 1 = play} |
| vocals:mute | Specify if the vocalization should be muted at the instructor interface. {0 = not muted ¦ 1 = muted} |

Media

| media:filename | Specify the filename of the media to be displayed. The filename includes base and extension and is assumed to be in the scenario’s media directory. NOTE: the filename is truncated to 63 characters on its way to the player. |
| --- | --- |
| media:play | Specify if the media file should be displayed. {0 = not displayed ¦ 1 = displayed} |

CPR

| cpr:compression | Percentage of chest displacement toward the ideal compressed position. 100 % is the ideal compressed position. |
| --- | --- |
| cpr:release (S) | Percentage of chest displacement toward the ideal released position. 0 % is the ideal released position. Reported by the controller. |
| cpr:duration (S) | Accumulated seconds of detected CPR within the current scene, reset at the beginning of each scene. Maintained by the engine. This is the correct trigger target for "CPR performed for at least N seconds". NOTE: <cpr><duration> written in an init section is accepted by the parser and then discarded. It is meaningful only inside a trigger. NOTE: cpr:duration is not currently published in the JSON status, so its value cannot be observed from an interface even though triggers can test it. |
| cpr:last (S) | Timestamp of the last detected compression start. |
| cpr:running (S) | Set while compressions are being detected. {0 = not running ¦ 1 = running} |

Pulse

| pulse:right_femoral (S) | Reports whether a right femoral pulse is being palpated, on a scale from 0 – 4, where 0 is no touch and 4 is the heaviest touch. Trigger target. |
| --- | --- |
| pulse:left_femoral (S) | Reports whether a left femoral pulse is being palpated, on a scale from 0 – 4. Trigger target. |
| pulse:right_dorsal (S) | Reports whether a right dorsal pulse is being palpated, on a scale from 0 – 4. NOTE: not available as a trigger target. |
| pulse:left_dorsal (S) | Reports whether a left dorsal pulse is being palpated, on a scale from 0 – 4. NOTE: not available as a trigger target. |
| pulse:duration (S) | Duration of the current pulse palpation in milliseconds. Trigger target. |
| pulse:active (S) | Indicates whether any pulse is being palpated. {0 = no ¦ 1 = yes} Trigger target. |

Telesim

The telesim class controls the remote video windows.  There are two windows, addressed as vid0 and vid1.  In XML the window index is the enclosing element:

<init>

<telesim>

<enable>1</enable>

<vid0>

<name>intake_video</name>

<command>1</command>

<param>0</param>

</vid0>

</telesim>

</init>

| telesim:enable | Enable the remote video windows. {0 = disabled ¦ 1 = enabled} NOTE: set this in the top-level <init> section. A <telesim> block inside a scene’s <init> does not work – see Section 12. |
| --- | --- |
| telesim:name | Name of the clip shown in window N, written as <vidN><name>clip</name></vidN>. N is 0 or 1. |
| telesim:command | Playback command for window N. {0 = stop ¦ 1 = start ¦ 2 = seek} |
| telesim:param | Parameter for the command in window N. For seek, the time in seconds; accepts a decimal value. |
| telesim:next | Index of the next clip to play in window N. |

Trigger Targets

A parameter trigger may only test a parameter that appears in the list below.  This is a strict subset of the API values in the preceding sections: a parameter that can be set is not necessarily one that can be tested.

A trigger naming a class or parameter that is not in this list evaluates to −1 rather than reporting an error, so a misspelled trigger target will silently never fire — or, with a test such as LT, will fire immediately and permanently.  Check spelling against this table.

| cardiac:rate | Current heart rate. |
| --- | --- |
| cardiac:avg_rate | Heart rate averaged by the engine. |
| cardiac:vpc_freq | VPC frequency, 0 – 100. |
| cardiac:vpc_delay | VPC delay. |
| cardiac:pea | Pulseless electrical activity, 0 or 1. |
| cardiac:arrest | Arrest indicator, 0 or 1. |
| cardiac:nibp_rate | Reported NIBP pulse rate. |
| cardiac:nibp_read | NIBP read in progress. |
| cardiac:nibp_linked_hr | NIBP rate linked to heart rate, 0 or 1. |
| cardiac:nibp_freq | Automatic NIBP interval in minutes. |
| cardiac:pr_interval | PR interval in milliseconds. |
| cardiac:qrs_interval | QRS interval in milliseconds. |
| cardiac:bps_sys | Systolic blood pressure. |
| cardiac:bps_dia | Diastolic blood pressure. |
| cardiac:ecg_indicator | ECG leads attached, 0 or 1. |
| cardiac:bp_cuff | Blood pressure cuff attached, 0 or 1. |
| cardiac:abp_line | Arterial line placed, 0 or 1. |
| cardiac:pac_placed | PA catheter placed, 0 or 1. |
| cardiac:pac_ra_mean | RA / CVP mean pressure. |
| cardiac:pac_rv_sys | RV systolic pressure. |
| cardiac:pac_rv_dia | RV diastolic pressure. |
| cardiac:pac_pa_sys | PA systolic pressure. |
| cardiac:pac_pa_dia | PA diastolic pressure. |
| cardiac:pac_wedge_mean | Wedge mean pressure. |
| respiration:rate | Set respiration rate. |
| respiration:awRR | Calculated airway respiration rate. May also be written awrr. |
| respiration:spo2 | SpO₂ percentage. |
| respiration:etco2 | etCO₂ in mmHg. |
| respiration:etco2_indicator | etCO₂ sensor connected, 0 or 1. |
| respiration:spo2_indicator | SpO₂ sensor connected, 0 or 1. |
| respiration:chest_movement | Chest movement enabled, 0 or 1. |
| respiration:manual_count | Manual breaths counted in this scene. |
| general:temperature | Temperature in tenths of a degree. |
| general:temperature_enable | Temperature probe connected, 0 or 1. |
| telesim:enable | Remote video enabled, 0 or 1. |
| cpr:duration | Accumulated seconds of CPR in this scene. |
| pulse:left_femoral | Left femoral palpation pressure, 0 – 4. |
| pulse:right_femoral | Right femoral palpation pressure, 0 – 4. |
| pulse:duration | Duration of the current palpation in milliseconds. |
| pulse:active | Any pulse being palpated, 0 or 1. |

The following are reported in the status but cannot be tested by a trigger: cardiac:rhythm, cardiac:vpc, cardiac:vfib_amplitude, cardiac:pwave, cardiac:abp_waveform, cardiac:pac_position, cardiac:heart_sound and its volume and mute, all four pulse strength parameters, respiration:co2_waveform, respiration:spo2_waveform, both lung sounds and their volume and mute, respiration:breathCount, respiration:inhalation_duration, respiration:exhalation_duration, general:temperature_units, cpr:compression, cpr:release, cpr:running, pulse:left_dorsal, pulse:right_dorsal, and the whole of the media, vocals, scenario and defibrillation classes.  A trigger on a string value is not supported at all; use an event trigger instead.

Known Engine Defects

The following are defects in the current engine, not properties of the schema.  They are recorded here because each one can make a correctly written scenario behave unexpectedly, and because a scenario author has no other way to know.  They should be removed from this section as they are fixed.

cardiac:cpr_time returns the wrong value

A trigger written as <cardiac><cpr_time>N</cpr_time></cardiac> is accepted, but the engine returns the blood pressure cuff state instead of any CPR timing.  There is no cpr_time field.  Use <cpr><duration> instead, which is the documented and working form.

A per-scene telesim block cannot enable video

A <telesim> block inside a scene’s <init> has its value mangled before it reaches the parser, so <enable> always evaluates to 0.  Enable telesim in the top-level <init> section instead, where it works correctly.

clock_start is overwritten

The value given in <general><clock_start> is copied into the status and then immediately overwritten with the current wall-clock time.  The element therefore acts as "reset the displayed clock to now" and cannot set a specific start time.

Two or more trigger groups in one scene corrupt memory

When a scene starts, the engine walks its trigger groups to clear the "met" flags but follows the wrong list pointer.  With a single trigger group in a scene this is harmless.  With two or more, it writes outside an allocated block and the flags of triggers already satisfied are never cleared, so a scene that is re-entered can never satisfy the group again.  Until this is fixed, use at most one <trigger_group> per scene.

Misspelled elements are discarded silently

The parser records a status code for every element it does not recognise, but that code is only printed in verbose mode and never raises an error count.  A misspelled element – <hart_sound>, <bps_systolic>, <pulse_strength> – is discarded with no diagnostic anywhere and the scenario loads and runs.  The shipped Example_Scenario contains eleven instances of <pulse_strength>, which is not a recognised element; the four side-specific names in Section 11.3 are the correct ones.

Only these conditions currently abort a load: a duplicate or missing scene id, a duplicate or missing event id, no start scene, and the removed <triggers_needed> element.  Even the duplicate checks run only in verbose or check-only mode.

Lists are capped at 100 entries

Scenes, events, a scene’s triggers, a scene’s trigger groups, and a group’s triggers are each held in a list capped at approximately 100 entries.  The 101st entry is dropped with a message to standard error, which is not visible in the packaged application, and without setting an error count.  The scenario then runs with parts silently missing.

Element name matching is inconsistent

Most elements are matched exactly, but init, timeout, triggers, trigger_group and the five telesim elements are matched on a prefix.  <initialise>, <timeouts>, <triggers_x>, <enabled> and <names> are therefore all accepted as if they were the canonical spelling.  Do not rely on this; write the canonical names.

Media and vocals filenames are truncated to 63 characters

Both are read from the XML into a 256-character buffer and then copied into a 64-character one on the way to the player.  Keep filenames short.

A rhythm change cancels a blood pressure trend set in the same block

Changing to or from a pulseless rhythm resets the cardiac transfer time to 0, and that happens before the systolic and diastolic values consume it.  A scene init that sets <rhythm>, <transfer_time> and <bps_sys> together therefore produces an immediate step in blood pressure rather than the requested ramp.  Set the rhythm in one scene and trend the pressure in the next, or accept the step.

Negative trigger bounds cannot be expressed

The value of a parameter trigger is split on the first hyphen to produce the two bounds of an INSIDE or OUTSIDE test.  A negative value is therefore read as a range: <rate>-1</rate> becomes the range −1 to 1.